南京邮电大学通达学院

毕业设计(论文)外文资料翻译

学    院：教育科学与技术学院

专　　业：  数字媒体技术

学生姓名：      袁鹏飞

班级学号：     13004330

外文出处：Pers Ubiquit Comput (2014) 18:535–542

附件：1.外文资料翻译译文；2.外文原文

指导教师评价： 1．翻译内容与课题的结合度： □ 优 □ 良 □ 中 □ 差 2．翻译内容的准确、流畅： □ 优 □ 良 □ 中 □ 差 3．专业词汇翻译的准确性： □ 优 □ 良 □ 中 □ 差 4．翻译字符数是否符合规定要求：□ 符合 □ 不符合 指导教师签名： 年 月 日

附件一

在基于OpenWrt的嵌入式Linux上实现有性价比的家庭照明控制系统

Cheong Ghil Kim • Kuinam J. Kim

摘要 近年来，温室的效应已大大增加，随着能源需求的增长，世界面临着严重的能源问题。同时，家庭自动化系统日益普及;基于云技术向智能家居发展。本文介绍了一种基于具有遥控功能的小型嵌入式系统的高性价比的家用节能系统。为此，该系统由基于嵌入式Linux的无线路由器构成，用于平台开发低成本的能量控制服务器和智能手机的远程灯光控制应用。通过将OpenWrt移植到与连接有LED的接口板连接的无线路由器上来实现原型系统。远程访问和GUI功能通过使用Apple iPhone的TCP/IP编程实现。通过智能电话和无线路由器之间的插座通信以及通过无线路由器和接口板之间的USB通信来验证远程控制系统的操作。实施结果表明，基于OpenWrt的无线路由器可以通过照明控制提供节能和安全的好处。

关键词 无线路由，节能 ，OpenWrt，智能手机；

1 介绍

近年来，温室的重要性显着增加，因为随着其需求的增长，世界正面临严重的能源问题。 在这种情况下，家庭自动化系统不断获得普及，此外，智能电网[2]智能家居技术的发展[2]。 一般来说，家庭自动化系统具有容易地访问和容易地控制家庭中的各种设备和设备的能力，包括通过有线和无线网络对家庭的所有出现的本地和远程监视。最近，住宅能源管理已成为一个积极的话题，并已引入了几项研究[2-4]。

在商业建筑中，照明占能源总成本的40％。降低能源消耗已成为主要目标。这是一种众所周知的解决方案，可以用更节能的照明源替代现有的灯具，如LED是减少这种大量能源使用的方式之一。更大的能量消耗水平来自于不需要时关闭灯光，优化光照水平以适应工人需求，并减少对照明能源的总体需求。改善系统范围内的照明控制是确保照明能量尽可能自动降低的最佳方式. 基于各种技术的照明控制解决方案已被证明可将商业和工业建筑的照明能源消耗降低高达70％。这些解决方案过去受到成本，复杂性和适用性的限制，但新的无线技术正在提供扩展照明控制能力的方法，并将其提供给更广泛的客户群[5,6]。特别是消费者无线网络技术以及智能手机和平板电脑等移动设备的最新进展使得用户能够拥有更智能高效的智能家居，其中对家电和灯具的所有控制进行管理。

如今，无线网络无处不在，即使在家庭802.11无线网络也很受欢迎，避免接线成本并为所有房间提供连接。 因此，我们可以轻松找到低成本的无线路由器，并且可以利用最初针对Linksys WRTG54的OpenWrt的低成本嵌入式Linux平台，但现在针对许多嵌入式无线设备，包括华硕，D-Link、NetGear、Soekris、Viewsonic和Linksys [7]。 主要地，OpenWrt是嵌入式设备用于路由网络流量的操作系统。 主要组件是Linux内核，uClibc和busybox。 所有组件都针对尺寸进行了优化，足够小以适应家庭路由器中有限的存储和内存[8]。

当今的典型家庭自动化系统包括Wi-Fi路由器或互联网连接，智能家庭网关和多个节点（称为终端设备）。 这些系统通常可以安装在标准的家庭中，使家庭变得聪明。 在本文中，一个小型住宅无线路由器被用作并入电源管理功能的智能网关，实现结果表明它可以显着降低家庭的功耗。 为此，我们利用OpenWrt的无线路由器作为具有遥控功能的家庭节能器。

多年来，由于电气和电子设备的数量在家庭和建筑物中大幅增加，由于光控制和照明分布效率低，难以管理能源的浪费[9]。另外，依靠用户直接控制灯开关节省能源是不现实的。 为此，本文介绍了基于OpenWrt的远程LED控制系统。 原型系统通过将无线路由器移植到OpenWrt并将接口板连接到LED来实现。 此外，该系统通过插座通信与智能手机通信以控制LED。

本文的结构如下，第2节，我们将提供有关家庭自动化系统和OpenWrt和章节的详细背景信息。第3节将介绍基于OpenWrt的家庭节能系统的建议信息。在第4节我们展示了实施过程及其结果。 第5节是结论。

2 背景

2.1 家庭自动化系统

家庭自动化系统的研究已经进行了很长时间。但是，在某种意义上说是正确的

这些技术尚未得到广泛采用，具有成本高，灵活性差，可管理性差和实现安全难度等几个障碍。即使在这种情况下，这一领域的研究也将该技术的能力扩大到远程监控，电力管理，跟踪和安全系统以及灾害预警系统等领域。对于示例系统，已经构建了Orange [11]，Aware Home [12]，House_n [13]，eHome [14]和基于Web的[15]，其中大多数已经实现了商业上可用的技术：Z- Wave [16]，ZigBee [17]和X10 [18]。在现在的日子里，特别是电源管理一直是特别有意义的，因为它可以让更环保的未来，并且作为削减成本的措施增加了用户的优势[19,20]。扩大家庭自动化领域的其他有趣研究可能包括老年人，机器人等问题[21-25]。

智能家居中的家庭自动化系统的核心功能是允许用户通过互联网本地或远程访问和控制家中的所有设备。为此，家庭中的所有设备必须具有通用接口并将其与系统和服务远程连接。图1显示了由轻型，媒体，HVAC，安全和户外控制组成的家庭自动化系统的一般框图。在这里，家庭网关将家庭周围的所有设备与系统的控制塔进行联网。

硬件设备的基本功能可以概括如下[20]。

2.1.1 家庭网关

家庭自动化系统中的网关的目的是实现所使用的不同技术之间的桥接。 它还可以作为在Wi-Fi路由器的帮助下将家庭系统连接到外部服务，反之亦然的手段。 家庭网关和家用电器之间的连接可以通过无线网络（如蓝牙，WIFI或有线网络，家庭PNA和IEEEI394）进行。

2.1.2 传感器

传感器可用于从家庭内部或外部的某些方面中继信息，以使系统适应发生的变化。 传感器与家庭网关直接通信，并提供关于特定房间内的光强度，家庭内外的温度和运动感测的系统信息。 此外，当房间短时间内没有检测到运动时，应该能够自动关闭设备。 照明系统允许足够的光线，无论何时何地用户需要它。 这是通过传感器的帮助完成的，传感器可以传递关于室内和/或室外的光强度的信息。

图 1 家庭自动化系统的一般架构

2.1.3 标准策略和设备

家里散布着许多设备。 根据家庭的大小和人数，家中发现的设备数量可能会有所不同。 还应该指出的是，由于人为疏忽，住房人数较多的家庭电力往往会更多。 它们可能需要不同的电量来运行，并且需要为每个设备找到具体的控制策略。

2.2 OpenWrt

OpenWrt是一个基于Linux的嵌入式平台，最初针对来自Linksys的具有Wi-Fi功能的Linksys WRTG54 [20]，但现在面向许多嵌入式无线设备，包括华硕，D-Link，NetGear，Soekris，Viewsonic和儿子的设备上。基本上，任何使用Infeon ADM5120，TI AR7，Intel IXP4xx系列或Broadcom BCN63xx系列的闪存和RAM的设备通常都可以运行OpenWrt。在某些情况下，功能可能会受到限制，具体取决于设备中的设备[7]。

因此，OpenWrt是嵌入式设备的高度可扩展的GNU / Linux发行版。它是由底层构建成为一个全功能，易于修改的小型网络设备操作系统[8]。 OpenWrt提供了类似于现代基于包的Linux发行版的功能：内置的支持CGI的Web服务器，一个SSH服务器，最重要的是一个包管理工具ipkg。使用ipkg，可以在运行时添加和删除新的应用程序，工具和内核驱动程序，而无需重新启动。 OpenWrt通过使用路由器提供的标准固件升级机制加载到预期路由器上。然而，从某种意义上来说，它并没有提供完整的桌面体验，它仍然提供了从现代Linux发行版预期的大部分功能[7]。

嵌入式设备上的空间是一个非常实际的约束，因为这个OpenWrt使用busybox作为shell环境。 BusyBox嵌入了许多常见的命令行工具，因此分发不必提供单独的可执行文件。这减少了所需的空间需求，允许OpenWrt在有限空间的设备上提供接近完整的Linux体验。安装OpenWrt后，路由器可以配置为无线接入点，无线客户端或自组织网络的成员。后一种选择允许创建网状网络，不需要网络基础设施。这对于试验紧急或植物行为是有用的。 OpenWrt具有不同的方法来构建固件，从头开始下载，修补和编译所有内容，包括交叉编译器。

OpenWrt不包含任何可执行文件，只包含很少的源代码;它是一个用于下载源的自动化系统，修补它们以与给定的平台一起工作，并为该平台正确编译它们。这意味着只要更改模板，就可以改变过程中的任何步骤[7]。

几乎任何标准的非GUI Linux / Unix程序都可以重新编译以用于OpenWrt，假设它可以适应目标路由器的较小内存占用。交叉编译和打包技术简单明了。软件安装使用与系统软件包相同的ipkg机制。系统和第三方软件包库都包含了越来越多的标准Linux工具和设备驱动程序列表[26]。

包括目录结构，软件包和外部存储库，工具链，软件架构在内的OpenWrt概述总结如下[27,28]。

2.2.1 目录结构

基础中有四个关键目录：工具，工具链，包和目标; 工具和工具链是指将用于构建固件映像，编译器和C库的常用工具。

2.2.2 软件包和外部存储库

在OpenWrt固件中，几乎所有的东西都是一个ipk，一个软件包，可以添加到固件中以提供新功能或删除以节省空间。 请注意，包裹也保留在主干外，可以使用包装进料系统从颠覆中获得。 这些软件包可以用于扩展构建系统的功能，并且需要链接到主干线。

2.2.3 工具链

OpenWrt通过在交叉编译过程中将正确的参数传递给编译器并使用已知可用于目标体系结构的修补程序来自动执行特定架构的工具链创建。 它还允许用户在gcc，binutils，内核头和uClibc的不同组合之间进行切换，以便用户可以轻松地查看回归和程序编译。 用户甚至可以自定义传递给交叉编译器的编译器标志，以便用户可以测试交叉编译器的优化功能或交叉编译库的不同位置

2.2.4 软件架构

OpenWrt使用常见的嵌入式Linux工具，如uClibc，busybox和shell解释器，并提供硬件抽象层和软件包管理器。 图2显示了OpenWrt使用的软件堆栈。

每个架构都使用不同的Linux内核，允许用户空间环境在设备之间共享和一致。 因此，用户只需要重新编译uClibc和程序包来匹配目标体系结构，以便在完全不同的嵌入式设备上运行相同的程序。

2.2.5统和包配置

采用统一配置接口的UCI，OpenWrt的范围与其他没有NVRAM的设备一起存储在单独的闪存分区中。 由于UCI是C库，因此可以轻松集成到现有的用户空间应用程序中，或开发与新应用程序兼容的OpenWrt配置存储。 UCI的进一步发展包括使用UCI作为配置文件格式的Web界面以及SNMP插件，以便轻松更改配置并对嵌入式设备执行操作。

| UCI | IPKG | User Programs |
| --- | --- | --- |
| Busybox |  |  |
| Uclibc |  |  |
| Linux Kernel |  |  |

图 2 OpenWrt的软件体系结构图

3 系统组件和配置

一般来说，家庭网关对家庭能源的控制必须具备传感，处理和联网的功能。图3 [29]显示了家庭自动化系统中的典型功率管理方案，包括家庭周围的所有电源，智能代理在其中控制具有不同操作环境的不同设备。

本文介绍了基于具有遥控特性的小型嵌入式系统的具有成本效益的家庭节能系统。为此，该系统由基于嵌入式Linux的无线路由器组成，用于开发低成本能量控制服务器和智能手机，用于远程灯光控制应用，如图1所示。原型系统通过将OpenWrt移植到与连接有LED的接口板连接的无线路由器上来实现。

该系统包括用于控制OpenWrt上的无线路由器，接口板和LED的智能手机以及与该板进行通信的无线路由器。这里，路由器是DLink的DIR-825 [30]无线路由器，作为CPU板来控制和管理系统。图5显示了其硬件和软件规格分别总结在表1和表2中。图5显示了DIR-825的路由器，图6描绘了PC和OpenWrt路由器之间的链路图[31]。

图 3 智能家居智能电源管理系统典型框图

苹果的智能手机iPhone4与无线路由器通信，并允许用户界面（UI）通过与其USB中安装的接口板进行通信来远程控制LED。对于接口板，我们使用Arduino实验室的Arduino UNO [28]。 Arduino Uno是基于ATmega328（数据表）的微控制器板。它有14个数字输入/输出引脚（其中6个可用作PWM输出），6个模拟输入，16 MHz陶瓷谐振器，USB连接，电源插孔，ICSP接头和复位按钮。它包含支持微控制器所需的一切;只需使用USB电缆将其连接到计算机，或者用AC到DC适配器或电池为其启动电源。图7显示了Arduino UNO接口板的图像，表3显示了规格。我们使用DLINK的DIR-825无线路由器和5欧姆红色LED。图4说明了使用iPhone远程控制LED的亮度的功能的系统的整体结构。

图 4 系统操作流程

图 5 DIR-825

表格 1  无线路由的详细信息

| 设施 | 描述 |
| --- | --- |
| 结构 | MIPS |
| 供应商 | Atheros |
| Bootloader | U-Boot |
| 芯片系统 | Atheros AR9132 |
| CPU速度 | 400 MHz |
| 闪存大小 | 32 MB |
| 内存大小 | 64 MB |
| 网络接口 | 4个 |
| 无线 | Atheros AR9160 BB/MAC and AR9103 2.4 GHz 3*3 MIMO radio b/g/n |
| 网卡 | RealTek RTL8366S Gigabit w/port-based |
| WLAN支持 | 支持 |
| USB | 是 |
| 串口支持 | 是 |

4 实施环境

4.1 交叉开发环境

为了开发使用无线路由器的应用，应该利用交叉开发环境。 这里，交叉编译器是能够为除编译器运行的平台之外的平台创建可执行代码的编译器。 通常，交叉编译器工具用于为嵌入式系统或多个平台生成可执行文件。 它用于编译一个不能执行编译的平台，如不支持操作系统的微控制器。 将系统可能具有一个或多个平台使用在半虚拟化中使用此工具变得越来越普遍[32]。

图8显示了使用OpenWrt的交叉编译环境。 交叉编译的执行是通过在导演完成编码之后执行make来分配交叉编译器的路径来执行的

4.2 OpenWrt的安装

实现建议的系统的第一步是用OpenWrt替换AP固件。 最简单的方法是使用AP上的内置固件升级网页上传OpenWrt固件。 但是，不支持这个功能。 第二种方法是通过TFTP安装固件。 当AP启动时，它运行一个启动程序，验证并加载实际的固件。 中断引导程序的方法取决于AP型号。

在路由器上安装OpenWrt的步骤按照以下顺序使用TFTP。

•连接路由器和PC

•释放窗口防火墙并激活FTTP

•下载最新的OpenWrt固件

•初始化网络采用者的IP

•ARP路由设置

•通过TFTP上传固件

•删除ARP路由信息

•改变网络用户的IP

•更改路由器的根密码

OpenWrt系统可以通过三种不同的方式进行配置。第一个是通过图形界面进行配置。要通过图形界面配置系统，应该使用任何Web浏览器。配置操作系统的另一种方法是配置命令行（终端）。第三种方法是编辑OpenWrt系统配置文件，它们存储在/etc/config /direc-tory中。要配置系统，可以使用内置的编辑器。

表格 2 安装环境

| 宿主机系统 | Window XP 以上 Linux Fedora 13 | 需要上传的内核版本Kernel 2.6.36 |
| --- | --- | --- |
| 路由 | BUFFALO WZR-HP-G300NH | 需要 |
| 固件 | OpenWrt Backfire10,03 | 最新的版本: 10.03.1-rc6 Backfire porting |
| 包 | TFTP | 固件安装 |

OpenWrt系统可以通过三种不同的方式进行配置。第一个是通过图形界面进行配置。要通过图形界面配置系统，应该使用任何Web浏览器。配置操作系统的另一种方法是配置命令行（终端）第三种方法是编辑OpenWrt系统配置文件，它们存储在/etc/config /direc-tory中。要配置系统，可以使用内置的编辑器。

图 6 OpenWrt路由器连接图

图 7 Arduino UNO

表格 3 Arduino UNO的详细信息

| 微型控制器 | ATmega328 |
| --- | --- |
| 工作电压 | 5 V |
| 建议输入电压 | 7 ~ 12 V |
| 输入电压极限 | 6 ~ 20 V |
| 数字I/O输出 | 14 (6个口提供PWM输出) |
| 模拟输出 | 6 |
| 每个I/O口直流 | 40 mA |
| 3.3V每个I/O口直流 | 50 mA |
| 闪存 | 32 KB (ATmega328) 其中0.5K被 bootloader使用 |
| 超级闪存 | 2 KB (ATmega328) |
| 只可读内存 | 1 KB (ATmega328) |
| 时钟速度 | 16 MHz |

4.3 OpenWrt的版本

稳定的版本已经出版，目前已经推出了三个主要版本的OpenWrt。

4.3.1	Whiterussian

Whiterussian遗留下来的版本仍然可用，但不支持或维护。 该固件基于Linux内核2.4，主要用于使用Broadcom CPU的路由器。

4.3.2	Kamikaze

为了支持许多CPU，引入了上风。 2006年10月，Buildroot-NG旗下的Buildroot-NG叉车大幅改善，并于2006年10月中旬并入Kamikaze主要开发部门，并成为Kamikaze首次正式发行。 有几个Kamikaze版本。

4.3.3 Backfire

第一次Backfire 10.03发布于2010年4月。维护版本10.03.1于2011年12月发布。然后态度调整12.09是新的稳定版本。

图 8 交叉编译

4.4 执行

首先，我们在路由器中安装了OpenWrt，然后安装了KMOD-USB-ACM软件包。接下来，我们将其连接到Arduino，以准备板和路由器之间的通信。 UI被应用于智能手机，并通过使用编程的iOS SDK来实现通过TCP / IP协议的套接字通信[33]。然后，我们使用C和Linux的库进行TCP / IP协议的套接字通信[34]，并通过USB端口设置用于控制数据传输的服务器程序的波特率。

我们选择了一个USB端口，并使用一个文件描述符，每个[35]发送数据1个字节，以便通过交叉编译适合OpenWrt系统。 Arduino的接口板通过USB端口从路由器的每个字节接收数据。当接收到所有数据时，我们对接口板进行编程，以将控制信号传送到LED [36]。最后，我们通过取决于控制信号的MOSFET开关来控制LED的亮度。

图9是系统整体结构图。我们创建了一个带LED的面包板，然后通过USB端口将其连接到主板。我们运行基于OpenWrt的软件，然后插入如图1所示的启动命令。因此，智能手机已准备好与无线路由器进行通信.

我们加载了UI，并从智能手机连接了无线路由器和TCP套接字通信，如图所示。 然后，我们通过数据传输来控制LED的亮度。 亮度可以从光（255）到暗（50）控制。 图9显示了智能手机上通过UI改变亮度值时LED亮度的变化。

图 9 原型实现

此外，在更改路由器的防火墙设置以允许外部路由器访问后，不仅本地无线路由器，而且还可以控制来自外部网络的3G互联网连接。

图 10 程序开始

5 结论

如今，随着能源需求的增长，世界一直面临严重的能源问题，温室气氛的意义大大增加。在这种情况下，本文介绍了一种可以节省家庭能源的低成本嵌入式系统。原型实现由控制LED灯制成。为此，我们实现了一个可以通过使用连接到由智能手机移植OpenWrt的无线路由器的接口板来控制LED的系统。这使得能够使用真实的无线路由器进行套接字通信并与外围设备链接，并且利用无线路由器作为嵌入式系统。本文结果的另一个应用可能包括通过互联网控制远程家庭自动化系统。如果使用这种系统，预计功耗将会降低，因为即使我们处于全球的对面，我们也可以通过互联网关闭家庭的灯光。此外，该系统可以通过使个人以更定制的方式控制不必要的亮LED的亮度来有助于节省功率。

附录二

Implementation of a cost-effective home lighting control system on embedded Linux with OpenWrt

Cheong Ghil Kim • Kuinam J. Kim

Abstract In recent years, the significance of greenhouses has been increased greatly because the world has been facing serious problems with energy as its growing demand. At the same time, home automation systems have been steadily gaining popularity; growing toward smart home based on Cloud technology. This paper introduces a cost-effective home energy saving system based on a small embedded system with remote controlling feature. For this purpose, the system is composed of a wireless router based on embedded Linux for the platform to develop a low-cost energy control server and a smart phone for remote light control app. The prototype system was implemented by porting OpenWrt onto the wireless router which is connected with an interface board with LEDs attached. The remote access and GUI function were implemented by TCP/IP programming using Apple iPhone. The operation of the remote control system was verified by socket communication between the smart phone and the wireless router, and by USB communication between the wireless router and the interface board. The implementation result shows that an OpenWrt-based wireless router can give benefits of saving energy and safety through lighting control.

Keywords    Wireless router • Energy saving  OpenWrt Smart phone

C. G. Kim (El)

Department of Computer Science, Namseoul University,

Cheonan, Choongnam, South Korea

e-mail: cgkim@nsu.ac.kr

K. J. Kim

Department of Convergence Security, Kyonggi University, Suwon-si, Gyeonggi-do, South Korea e-mail: harapl23@hanmail.net

1 Introduction

In recent years, the significance of greenhouses has been increased greatly because the world has been facing serious problems with energy as its growing demand. In connection with this situation, home automation systems have been steadily gaining popularity, furthermore, growing toward smart home [1] technology in the smart grid [2]. In general, home automation systems have the functions of easy accessibility and convenient control of various appliances and devices in the home including monitoring all events happening at home both locally and remotely via wired and wireless networks. Recently, residential energy management has become an active topic and several researches have been i ntroduced [2-4].

In commercial buildings, lighting accounts for up to 40 % of total energy cost. Reducing this energy consumption has become a major goal. It is a well-known solution that to replace existing lights with more energy-efficient lighting sources such as LED is one of the ways to reduce this massive pool of energy use. An even greater level of energy reduction comes from turning off lights when they are not needed, optimizing light levels to suit worker needs and reducing overall demand for lighting energy. Improving system-wide control over lighting is the best way to ensure that lighting energy is automatically reduced as much as possible. Lighting control solutions, based on a variety of technologies, have been proven to reduce lighting energy consumption in commercial and industrial buildings by up to 70 %. These solutions have been limited in the past by cost, complexity and applicability, but new wireless technologies are providing ways to expand the capabilities of lighting control and offer them to a wider set of customers [5, 6]. Especially, recent advances in consumer wireless networking technologies and mobile devices such as smart phones and tablets are enabling users to have more intelligent and efficient smart home in which all controls of appliance and light are managed.

Nowadays, wireless networks are everywhere and even in home 802.11 wireless networks are popular, avoiding wiring costs and providing connectivity for all rooms. Therefore, we can find low-cost wireless router easily, and they can be utilized for low-cost embedded Linux platform using OpenWrt which targeted at the Linksys WRTG54 initially, but now targets many embedded wireless devices including equipment from Asus, D-Link, NetGear, Soekris, Viewsonic, and Linksys [7]. Primarily, OpenWrt is an operating system used on embedded devices to route network traffic. The main components are the Linux kernel, uClibc and busybox. All components have been optimized for size, to be small enough to fit the limited storage and memory available in home routers [8].

A typical home automation systems of today consist of a Wi-Fi router or an internet connection, a smart home gateway and multiple nodes (known as end devices). These systems can commonly be installed in standard home, making the homes smart. In this paper, a small residential wireless router is used as smart gateways incorporating power management features and the implementation result shows its feasibility of substantially reducing the power consumption of a home. For this purpose, we take advantage of the wireless router with OpenWrt as the home energy saver with remote control feature.

Over the years, as the number of electric and electronic devices increases dramatically at home and in buildings, it was difficult to manage the waste of energy due to the inefficient light control and illumination distribution [9]. In addition, it is not practical to rely on users to directly control the light switch to save energy. To this end, this paper introduces a remote LED control system which is based on OpenWrt. The prototype system was implemented by porting a wireless router to OpenWrt and by connecting an interface board to an LED. Also, the system communicated with a smart phone via socket communication to control LED.

The paper is structured as follows, in Sect. 2 we will provide detailed background information on home automation systems and OpenWrt and Sect. 3 will introduce information regarding the proposed home energy saving systems based on OpenWrt. In Sect. 4 we show the procedures of implementation and its result. Section 5 covers the conclusion.

2 Background

2.1 Home automation systems

The researches on home automation systems have been conducted for long times. However, it is true in some sense

that these technologies have not been widely adopted with several barriers such as high cost of ownership, inflexibility, poor manageability and difficulty achieving security [10]. Even under this circumstance, researches in this field have extended the enhanced capabilities of the technology into areas such as remote monitoring and control, power management, tracking and security systems and disaster warning systems. For examples systems, Orange [11], Aware Home [12], House_n [13], eHome [14], and Web-based [15] have been built, and most of them have been implemented with technologies commercially available: Z-Wave [16], ZigBee [17] and X10 [18]. In now days, especially, power management has been of particular interest as it allows for a greener future and is added advantage to users as a cost-cutting measure [19, 20]. Other interesting research that broadens the areas of home automation may include the issues of elderly people, robotics and so on [21-25].

The core function of home automation systems in smart home has to allow users to access and control all the devices at home either locally or remotely via the internet. For this purpose, all devices at home have to have common interface and link it to systems and services remotely. Figure 1 shows a general block diagram of home automation systems consisting of with light, media, HVAC, security and outdoor controls. Here, a home gateway is networking all devices around home as the control tower of the system.

Basic functions of hardware devices may be summarized as bellowing [20].

2.1.1 Home gateway

The purpose of a gateway in a home automation system is to enable bridging between different technologies used. It also acts as a means to connect the home system to external services and vice versa with the help of a Wi-Fi router. Inter-connections between home gateway and home appliances are possible through wireless networks such as Bluetooth, WIFI or wired network, Home PNA and IEEEI394.

2.1.2 Sensors

Sensors can be used to relay information on certain aspects from within or outside the home to enable the system to adapt to the occurring changes. Sensors communicate directly with the home gateway and feed the system information with regards to light intensity inside a particular room, temperature inside and outside the home and motion sensing to name a few. Also, it should be able to automatically turn off devices when it detects no motion in the room for a brief period of time. Lighting systems allow just enough light wherever and whenever the user requires it. This is done with the help of sensors that relay information regarding the intensity of the light inside and/or outside the house.

Figure 1  General architecture of home automation systems

2.1.3 Standard appliance and devices

There are many devices scattered around a home. Depending on the size and number of occupants in a home, the number of devices found in homes can vary. It should also be noted that there tends to be more wastage in terms of electricity in homes with a higher number of occupants due to human negligence. They may require different amounts of power to run, and it is necessary to find specific control policy for each device.

2.2 OpenWrt

OpenWrt is a Linux-based embedded platform initially targeted at the Linksys WRTG54 [20], Wi-Fi capable res¬idential gateway from Linksys, but now targets many embedded wireless devices including equipment from Asus, D-Link, NetGear, Soekris, Viewsonic and son on. Basically, any equipment with adequate flash memory and RAM that uses the Infeon ADM5120, TI AR7, Intel IXP4xx series or the Broadcom BCN63xx series can typically run OpenWrt. The functionality may be limited in some instances, depending on the equipment in the device [7].

Therefore, OpenWrt is a highly extensible GNU/Linux distribution for embedded devices. It is built from the

ground up to be a full-featured, easily modifiable operating system for small network devices [8]. OpenWrt provides features similar to a modern package-based Linux distri¬bution: a built-in web server with CGI support, an SSH server and most importantly, a package management tool, ipkg. Using ipkg, new applications, tools and kernel drivers can be added and removed at runtime, without requiring a reboot. OpenWrt is loaded onto the prospective router by using the standard firmware upgrade mechanism provided by the router. However, in the sense that it does not provide a complete desktop experience, it still provides much of the functionality expected from a modern Linux distribution [7].

Space on embedded devices is a very real constraint, because of this OpenWrt uses busybox as the shell envi¬ronment. BusyBox embeds many of the common command line tools, so a distribution does not have to provide indi¬vidual executables. This cuts down on the space require¬ments needed allowing OpenWrt to provide a near complete Linux experience on devices with limited space. With OpenWrt installed, the router can be configured as a wireless access point, a wireless client or a member of an ad hoc network. The latter option allows mesh networks to be created, requiring no networking infrastructure. This is useful for experimenting with emergent or flocking behaviors. OpenWrt has different approach to building a firmware, downloading, patching and compiling everything from scratch, including the cross-compiler.

OpenWrt does not contain any executables and only very few sources; it is an automated system for down¬loading the sources, patching them to work with the given platform and compiling them correctly for that platform. What this means is that just by changing the templates, you can change any step in the process [7].

Almost any standard non-GUI Linux/Unix program can be recompiled for use in OpenWrt, assuming it can fit in the smaller memory footprint of the target router. The cross-compilation and packaging techniques are simple and well documented. Installation of software uses the same ipkg mechanism as for system packages. Both the system and the third-party package libraries contain a growing list of standard Linux tools and device drivers [26].

The overview of OpenWrt including directory structure, packages and external repositories, toolchain, software architecture, is summarized as followings [27, 28].

2.2.1 Directory structure

There are four key directories in the base: tools, toolchain, package and target; tools and toolchain refer to common tools which will be used to build the firmware image, the compiler, and the C library.

2.2.2	Packages and external repositories

In an OpenWrt firmware, almost everything is an ipk, a software package which can be added to the firmware to provide new features or removed to save space. Note that packages are also maintained outside of the main trunk and can be obtained from subversion using the package feeds system. Those packages can be used to extend the func¬tionality of the build system and need to be linked into the main trunk.

2.2.3	Toolchain

OpenWrt automates the toolchain creation for a particular architecture by passing the right arguments to the compiler during the cross-compilation process and using patches known to be working for the target architecture. It also allows users to switch between different combinations of gcc, binutils, kernel headers and uClibc, so that users can easily check regressions and programs compilation. Users can even customize the compiler flags passed to cross-compiler so users can test optimizations features of your cross-compiler or different locations for your cross-com¬piled libraries.

2.2.4	Software architecture

OpenWrt uses the common embedded Linux tools such as uClibc, busybox and shell interpreter and provides a hardware abstraction layer and package manager. Figure 2 shows the software stack that OpenWrt uses.

Every architecture uses a different Linux kernel allow¬ing the user-space environment to be shared and consistent across devices. Therefore, users only need to recompile the uClibc and packages to match your target architecture to get the same programs running on a totally different embedded device.

2.2.5	System and package configuration

UCI which stands for Unified Configuration Interface was adopted with the extent of OpenWrt to other devices which did not have the NVRAM to store their settings into a separate flash partition. Since UCI is a C library, it can be easily integrated into an existing user-space application or to develop configuration storage that is OpenWrt compat¬ible for new applications. Further developments for UCI include a web interface that uses UCI as a configuration file format as well as SNMP plugins to easily change the configuration and take actions on the embedded device.

| UCI | IPKG | User Programs |
| --- | --- | --- |
| Busybox |  |  |
| Uclibc |  |  |
| Linux Kernel |  |  |

Figure 2  Software architecture of OpenWrt

3 System components and configuration

In general, for home energy control, a home gateway has to equip with the functions of sensing, processing and net¬working. Figure 3 [29] shows a typical power management scheme in home automation system including all power sources around home in which intelligent agents are con¬trolling different devices with different operation environments.

This paper introduces a cost-effective home energy saving system based on a small embedded system with remote controlling feature. For this purpose, the system is composed of a wireless router based on embedded Linux for the platform to develop a low-cost energy control server and a smart phone for remote light control app as shown in Fig. 4. The prototype system was implemented by porting OpenWrt onto the wireless router which is connected with an interface board with LEDs attached.

The system comprises of a smart phone to control a wireless router on OpenWrt, an interface board and LEDs, and a wireless router to communicate with the board. Here, the router is DIR-825 [30] wireless router of DLink and acts as a CPU board to control and manage the system. Figure 5 shows its hardware and software specifications are summarized in Tables 1 and 2, respectively. Figure 5 shows the router of DIR-825, and Fig. 6 depicts the link diagram between PC and OpenWrt router [31].

Figure 3  Typical block diagram of smart power management systems

The smart phone, iPhone4 of Apple, communicates with the wireless router and allows the user interface (UI) to control the LED remotely through the communication with the interface board installed in its USB. For the interface board, we use Arduino UNO [28] of Arduino Lab. The Arduino Uno is a microcontroller board based on the ATmega328 (datasheet). It has 14 digital input/output pins (of which 6 can be used as PWM outputs), 6 analog inputs, a 16 MHz ceramic resonator, a USB connection, a power jack, an ICSP header and a reset button. It contains everything needed to support the microcontroller; simply connect it to a computer with a USB cable or power it with an AC-to-DC adapter or battery to get started. Figure 7 shows the image of Arduino UNO interface board, and Table 3 shows the   specification. We used DIR-825 wireless router of DLink and 5 ohm red LEDs. Figure 4 illustrates the overall structure of the system with the functionality of controlling the brightness of LEDs using iPhone remotely.

Figure 4  System operating flow

Figure 5  DIR-825

Table 1 Specification of wireless router

| Items | Descriptions |
| --- | --- |
| Architecture | MIPS |
| Vendor | Atheros |
| Bootloader | U-Boot |
| System-on-chip | Atheros AR9132 |
| CPU speed | 400 MHz |
| Flash size | 32 MB |
| RAM | 64 MB |
| Network | 4 |
| Wireless | Atheros AR9160 BB/MAC and AR9103 2.4 GHz 3*3 MIMO radio b/g/n |
| Ethernet | RealTek RTL8366S Gigabit w/port-based |
| WLAN | Support |
| USB | Yes |
| Serial | Yes |

4 Implementation environments

4.1 Cross-development environment

In order to develop application using wireless routers, cross-development environment should be utilized. Here, a cross-compiler is a compiler capable of creating executable code for a platform other than the one on which the compiler is running. In general, cross-compiler tools are used to generate executables for embedded system or multiple platforms. It is used to compile for a platform upon which it is not feasible to do the compiling, like microcontrollers that do not support an operating system. It has become more common to use this tool for paravirtualization where a system may have one or more platforms in use [32].

Figure 8 shows the cross-compiling environment using OpenWrt. The execution of cross-compiling is conducted by executing make after finishing coding in a director and assigning the path of cross-compiler.

4.2 Installation of OpenWrt

The first step to implementing the proposed system is to replace the AP firmware with OpenWrt. The easiest method is to use the built-in firmware upgrade web page on the AP to upload the OpenWrt firmware. However, not all devices support this function. The second method is to install the firmware via TFTP. When the AP boots, it runs a bootloader which validates and loads the actual firmware. The method to interrupt the bootloader depends on the AP model.

The procedure for installing OpenWrt on the router follows the bellowing sequence using TFTP.

•	Connecting the router and PC

•	Release window firewall and activate FTTP

•	Download the newest OpenWrt firmware

•	Initialize the IP of network adopter

•	ARP route setting

•	Firmware upload through TFTP

•	Remove information of ARP route

•	Change the IP of network adopter

•	Change the root password of the router

Open WRT system can be configured in three different ways. The first is the configuration via graphical interface. To configure the system via a graphical interface one should use any web browser. Another way to configure the operating system is via configuring the command line (terminal). The third way is to edit OpenWrt system configuration files, which are stored in/etc./config/direc-tory. To configure the system it is possible to use built-in editor.

Table 2  Installation environment

| Host PCs OS | Window XP or later Linux Fedora 13 | Required for firmware upload Kernel 2.6.36 |
| --- | --- | --- |
| Router | BUFFALO WZR-HP-G300NH | Required |
| Firmware | OpenWrt Backfire10,03 | The newest version: 10.03.1-rc6 Backfire porting |
| Package | TFTP | Firmware installation |

Figure 6  OpenWrt router connection diagram

Figure 7  Arduino UNO

Table 3 Specification of Arduino UNO

| Microcontroller | ATmega328 |
| --- | --- |
| Operating voltage | 5 V |
| Input voltage (recommended) | 7 ~ 12 V |
| Input voltage (limits) | 6 ~ 20 V |
| Digital I/O pins | 14 (of which 6 provide PWM output) |
| Analog input pin | 6 |
| DC current per I/O pin | 40 mA |
| DC current for 3.3 V pin | 50 mA |
| Flash memory | 32 KB (ATmega328) of which 0.5 KB used by bootloader |
| SRAM | 2 KB (ATmega328) |
| EEPROM | 1 KB (ATmega328) |
| Clock speed | 16 MHz |

OpenWrt system can be configured in three different ways. The first is the configuration via graphical interface. To configure the system via a graphical interface, one should use any web browser. Another way to configure the operating system is via configuring the command line (terminal). The third way is to edit OpenWrt system configuration files, which are stored in/etc./config/direc-tory. To configure the system, it is possible to use built-in editor.

4.3 OpenWrt versions

Stable release versions have been made, and currently, three major versions of OpenWrt have been introduced.

4.3.1	Whiterussian

The legacy Whiterussian releases are still available, but are neither supported nor maintained. The firmware is based on Linux kernel 2.4 and mostly for the routers using Broad¬com CPU.

4.3.2	Kamikaze

Kamikaze was introduced in order to support many CPUs. Substantial improvements to the build environment were made under the Buildroot-NG fork in 2006, and these were merged back into the main Kamikaze development branch in mid-October 2006 and became the first official Kami¬kaze release. There were several Kamikaze releases.

4.3.3 Backfire

First Backfire 10.03 release was in April 2010. The maintenance release 10.03.1 was released in December 2011. And then attitude adjustment 12.09 is the new stable release.

Figure 8  Cross-compiling

4.4 Execution

First of all, we installed OpenWrt in the router and then KMOD-USB-ACM package. Next, we connected it to Arduino in order to prepare communication between the board and router. The UI was applied to the smart phone and operated to enable socket communication through TCP/IP protocol by using programmed iOS SDK [33]. Then, we used the library of C and Linux for socket communication by TCP/IP protocol [34], and set the Baud ratio of the server program for control data transfer by USB port.

We selected a USB port and used a file descriptor to send data by 1 byte each [35], so that it fits to the OpenWrt system by doing cross-compile. The interface board of Arduino received data by each byte from the router through USB port. We programmed the interface board to deliver the control signal to the LED when all data were received [36]. Finally, we controlled the brightness of the LED by the MOSFET switch that depends on the control signal.

Figure 9 is the picture of the overall structure of the system. We created a breadboard with LED and then connected it to the main board with the USB port. We ran the OpenWrt-based software and then inserted command for starting as in Fig. 10. As a result, the smart phone became ready to communicate with the wireless router.

We loaded the UI and connected the wireless router and TCP socket communication from the smart phone as shown in Fig. 4. Then, we controlled the brightness of LEDs by data transfer. The brightness is controllable from light (255) to dark (50). Figure 9 shows the change of brightness of LED as changing brightness values through UI on smart phone.

Figure 9  Prototype implementation

Moreover, after changing the firewall settings of the router to allow external router access, not only local wireless router, but also control of 3G internet connection from external network became possible.

Figure 10 Program start

5 Conclusion

Nowadays, the significance of greenhouses has been increased greatly because the world has been facing serious problems with energy as its growing demand. Under this circumstance, this paper introduces a low-cost embedded system that can save home energy. The prototype imple¬mentation was made by control LED lights. For this pur¬pose, we implemented a system that can control LED by using an interface board which was connected to a wireless router ported with OpenWrt by a smart phone. This enabled socket communication and link with peripheral device using a real wireless router, and utilization of a wireless router as an embedded system. Another application of the results of this paper may include control of remote household automation system via internet. If such system is utilized, it is expected that power consumption will be reduced, since we can turn off the lights in our homes via the internet even when we are on the opposite side of the globe. Furthermore, this system can contribute to saving power by enabling individuals to control the brightness of unnecessarily bright LEDs in a more customized fashion.

Acknowledgments

Funding for this paper was provided by Nam- seoul University.

References

- Haines V, Mitchell V, Cooper C, Maguire M (2007) Probing user values in the home environment within a technology driven smart home project. Pers Ubiquitous Comput 11(5):349-359

- Melike EK (2011) Wireless sensor networks for cost-efficient residential energy management in the smart grid. Smart Grid, IEEE Trans 2:314-325

- Dhiren T, Al-Kuwari AMAH, Potdar V (2011) Energy conservation in a smart home. In: Digital ecosystems and technologies conference (DEST). 2011 Proceedings of the 5th IEEE International Conference on, pp 241-246

- Corno F, Razzak F (2012) Intelligent energy optimization for user intelligible. Smart Grid, IEEE Trans 3(4):2128-2135

- Daintree Networks Inc (2010) The value of wireless lighting control.www.daintree.net. Accessed 17 Mar 2013

- Rand P (2013) Strategic marketing wireless lighting control: the bright road ahead. Low power RF IEEE 802.15.4 and ZigBee products, Texas instruments incorporated (TI) http://www.eetimes. com/ContentEETimes/Documents/TI%20paper.pdf. Accessed 17 Mar 2013

- SANS Institute, Murray J (2009) An inexpensive wireless IDS using Kismet and OpenWRT http://www.sans.org/reading_room/ whitepapers/detection/inexpensive-wireless-ids-kismet-open wrt_ 33103. Accessed 17 Mar 2013

- Wikipedia (2013) OpenWrt. http://en.wikipedia.org/wiki/Open Wrt. Accessed 17 Mar 2013

- Lee J (2011) IEEE 1451 interface for smart grid. Dissertation, Hanyang University Graduate School

- Brush AJB, Lee B, Mahajan R, Agarwal S, Saroiu S, Dixon C (2012) Home automation in the wild: challenges and opportunities. In: Sensing technology (ICST), 2012 sixth international conference on, ppl42-145

- Mozer MC (2005) Lessons from an adaptive house. In: Cook D, Das R (eds) Smart environments: technologies, protocols, and applications. Wiley, Hoboken, pp 273-294

- Kietz J, Patel S, Jones B, Price E, Mynatt E, Abowd A (2008) The georgia tech aware home. Ext. Abstracts CHI, pp 3675-3680

- Intille S (2002) Designing a home of the future. IEEE Pervasive Comput l(2):80-86

- Koskela T, Vaananen-Vaninio-Mattila K (2004) Evolution towards smart home environments: empirical evaluation of three user interfaces. Pers Ubiquitous Comput 8(3^1):234-240

- Selcuk GH, Muharrem G (2010) Web based ZigBee enabled home automation system. In: 13th international conference on network-based information systems (NBiS) 2010, pp 290-296

- Z Wave World, Inc (2007) Ask the expert. http://www. zwaveworld.com/ask/ask8.php. Accessed 17 Mar 2013

- ZigBee alliance (2008) ZigBee specification. http://www.zigbee. org/Specifications.aspx. Accessed 17 Mar 2013

- Rye D (2013) The X-10 POWERHOUSE power line interface model #PL513 and two-way power line interface model # TW523. ftp://ftp.xlO.com/pub/manuals/technicalnote.pdf. Accessed 17 Mar 2013

- Aman S, Simmhan Y, Prasanna VK (2013) Energy management systems: state of the art and emerging trends. Commun Mag, IEEE51(1):114-119

- O¨zden T, Okumu HI (2012) Designing a load agent for powermanagement with a multi-agent home automation system. In:Innovations in intelligent systems and applications (INISTA),2012 international symposium on, pp 1–5

- Nazabal JA, Matias IR, Fernandez-Valdivielso C, Falcone F,Branchi P, Mukhopadhyay SC (2012) Home automation basedsensor system for monitoring elderly people safety. In: The sixthinternational conference on sensing technology (ICST 2012),pp 142–145

- Sa IK, Ahn HS, Yi KM, Choi JY (2009) Implementation of homeautomation system using a PDA based mobile robot. In: Indus-trial electronics, 2009. ISIE 2009. IEEE international symposiumon, pp 1761–1766

- Li N, Yan B, Chen G, Govindaswamy P, Wang J (2010) Designand implementation of a sensor-based wireless camera system forcontinuous monitoring in assistive environments. Pers UbiquitousComput 14(6):499–510

- Chang YS, Wang WJ, Hung YS (2013) A near field communi-cation-driven home automation framework. Pers Ubiquitous Comput 17(1):169–185

- Tolmie P, Crabtree A, Egglestone S, Humble J, Greenhalgh C, Rodden T (2010) Digital plumbing: the mundane work of deploying UbiComp in the home. Pers Ubiquitous Comput 14(3):181-196

- Kurt TE (2006) Low-cost on-board linux, vision, wi-fi, and more for the roomba robotics base http://www.aaai.org/Papers/ Symposia/Spring/2007/SS-07-09/SS07-09-019.pdf. Accessed 17 Mar 2013

- Fainelli F (2008) The OpenWrt embedded development framework. http://downloads.openwrt.org/people/florian/fosdem/fosdem.pdf. Accessed 17 Mar 2013

- Arduino (2013) http://arduino.cc/en/Main/arduinoBoardUno. Accessed 17 Mar 2013

- Wikipedia (2013) Linksys WRT54G series http://en.wikipedia. org/wiki/Linksys_WRT54G_series. Accessed 17 Mar 2013

- OpenWrt:  D-link DIR-825  (2013) http://wiki.openwrt.org/toh/ d-link/dir-825. Accessed 17 Mar 2013

- Jeong GS, Kim CG, Kwak HG, Jang H (2009) Embedded linux systems and applications using wireless router (ISBN-10: 8957271309). Green, Seoul, Korea

- Wikipedia (2013) Paravirtualization. http://en.wikipedia.org/wiki/Paravirtualization. Accessed 17 Mar 2013

- MarkD, Nutting J, LaMarche J (2010) Beginning iPhone 5 development: exploring the iOS 5 SDK. Wikibooks, pp 259–366

- Yoon S (2007) TCP/IP programming. Orangemedia, pp 110–112

- Arduino forum (2013) UNO serial latency. http://arduino.cc/forum/index.php?topic=96. Accessed 17 Mar 2013

- Arduino playground (2013) Arduino MOTOR/LED control.http://arduino.cc/playground/MotorControlShieldV3/0. Accessed 17 Mar 2013